AT-OPUS Series Read Me

- To import the profile into your custom profiles library please see the KB Article on the Savant community on “*App Note* Profile Import Tool Setup Guide”

- After importing the profile into your custom library you may search for the profile under AT-OPUS-810M, 68M, or 46M

- Place the component into your blueprint and name it

- Next locate the AT-OPUS-RX and place this component in your blueprint for each Receiver you have installed in your system.

- Now begin making your connections for your HDMI and Audio inputs

- Now begin making your output connections to your AT-OPUS-RX devices and then to your display devices.

- Make the connection from the “ETHERNET” port to a network switch and enter the IP address of the AT-OPUS-XXX when prompted to

- If needed make connections from the Audio out to powered speakers or amplifiers.

- Once all your connections are made you have completed setting up the OPUS unit in the savant config.

NOTE:

You MUST you the proper model number OPUS profile to ensure proper audio routing as the input and output numbers change as the matrix gets larger.

See the Help video on youtube here: https://youtu.be/u9tOyj77l5Q