Products Engineering

Specification

- Table of Contents

1	Abstract	3

2	Disclaimer	3

2.1	Manufacture	3

2.2	Installation, testing, and system’s acceptance	3

3	Specifications summary	4

3.1	HDVS-200-TX / PSK Product Specifier	5

3.2	HDVS-200-TX-WP Product Specifier	6

3.3	HDVS-200-RX Product Specifier	7

3.4	HDVS-150-TX Product Specifier	8

3.5	HDVS-150-TX-WP Product Specifier	9

3.6	HDVS-150-RX Product Specifier	10

3.7	UHD-EX-100CE-RX-PSE Product Specifier	11

3.8	UHD-EX-100CE-TX-PD Product Specifier	12

3.9	AT-HDVS-300-KIT Product Specifier	13

3.10	AT-UHD-SW-5000ED Product Specifier	14

3.11	AT-UHD-SW-52ED Product Specifier	15

3.12	AT-HD-SC-500 Product Specifier	16

3.13	AT-UHD-CLSO-601 Product Specifier	17

3.14	AT-UHD-CLSO-612ED	17

3.15	AT-UHD-CLSO-824	18

3.16	AT-UHD-CLSO-840	19

3.17	OmniStream VC-2 (AT-OMNI-111, AT-OMNI-112, AT-OMNI-121, AT-OMNI-122)	21

3.17.2	Video Encoder and Decoder Specification	21

3.17.3	Audio Sub System Specifications	21

3.17.4	System	22

3.17.5	Hardware	24

3.17.6	Environmental	25

4	Definitions and Abbreviations	26

# Abstract

The objective of this document is to serve as an engineering source or Specifier for Altona’s products, to help the designer in their project specification. The document is provided as a convenience to our customers and will require editing and customization to meet the project scope and requirements.

# Disclaimer

Atlona Technologies shall not be liable for any damages arising out of using this guide. To insure accuracy of your design, please contact our design and support team at (800)-536-3976.

## Manufacture

Basis of design subject to compliance with requirements to provide products of Atlona Technologies head quartered at 70 Daggett Drive, San Jose CA 95134. U.S. Office: +1.877.536.3976, or international +1.408.962.0515

## Installation, testing, and system’s acceptance

Product installation, testing, customer training, and system acceptance shall be done by an Atlona Certified Partner/ Engineer.

# Specifications summary

The AV system and devices shall meet the following as minimum for product operation:

- The UHD/4K, and HD Digital Transport System shall operate as a stand -alone point to point system, as well as a part of larger switching system for delivering local AV content to a far end sinks.

- Audio/Video switching and distribution at native resolution without compression.

- HDMI signal transport

- Audio signal transport

- CEC support

- RS232 support

- HDCP 1.4 or greater support

- Deep Color support.

- EDID pass through and or EDID management.

- Display control support

- Auto signal detection

- Field Firmware upgradable

- Backed up with 10 years limited product warranty

- Specification

## HDVS-200-TX / PSK Product Specifier

The 3X1 switcher / transmitter shall have two HDMI and one VGA inputs, as well as audio on 3.5 connector. The transport shall extend HDMI, VGA (including audio), RGBHV, YUV, and Composite, as well as RS-232 and Ethernet over HDBaseT port. The switching shall have input select, as well as display and volume control buttons on the front panel. Device configuration, operation and, maintenance shall be available via software management and Web GUI access. When two or more signal inputs are available, the transmitter shall include integrated switcher with signal sensing. The switcher shall switch to the last detected input (when not used with a control system by the same manufacturer). The 4K /UHD Input Switcher shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty

- 3X1 HDBaseT Switcher - two HDMI inputs plus a VGA input with 3.5mm audio connector

- Front panel input select button

- Front panel display on/off control

- Front panel volume control

- Front LED indication for PWR, Input source selected, and signal status

- Video resolution support for 4096×2160@24/25/30/50*/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control: Selects active input when sources are connected

- Automatic input selection using video detection technology

- EDID management support

- CEC support

- HDCP 1.4 support

- PCM 2CH input audio

- PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio for HDMI IN and HDBaseT OUT

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- Transmits IP and AV signals up to 328 ft. (100m)

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- CE and FCC certified

## HDVS-200-TX-WP Product Specifier

The 2X1 switcher/ transmitter shall be able to extend HDMI, VGA (including audio), RGBHV, YUV, and Composite over HDBaseT port. The 2X1 switcher / transmitter shall have input select, as well as display and volume control buttons on the front panel. Device configuration, operation and, maintenance shall be available via software management and Web GUI access. When two signal inputs are available, the transmitter shall include integrated switcher with signal sensing.  The switcher shall switch to the last detected input (when not used with a control system by the same manufacturer). The 4K /UHD Input Switcher shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty.

- US, 2-gang Decora-style wall plate

- 2X1 HDBaseT Switcher - One HDMI input plus a VGA input with 3.5mm audio connector

- Front panel input select button

- Front panel display on/off control

- Front panel volume control

- Front LED indication for PWR and signal status

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control: Selects active input when sources are connected

- Automatic input selection using video detection technology

- EDID management support

- CEC support

- HDCP 1.4 support

- PCM 2CH input audio

- PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio for HDMI IN and HDBaseT OUT

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- Transmits IP and AV signals up to 328 ft. (100m)

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- Power consumption up 30W

- CE and FCC certified

## HDVS-200-RX Product Specifier

The HD Scaling receiver shall accept HDBaseT audio and video Signal, and outputs HDMI, as well as de-embedded balanced audio output. It shall have two RS-232 ports, and contact closure port for screen or display lift control. The scaler receiver shall have automatic display control via CEC or control system, as well as menu and volume adjustment via software management, web GUI, or front panel button control. The HD scaling receiver shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty.

- HD video scaler with HDMI output and input resolution control

- Automatic display control - Automatically changes projector power state based on active or standby mode of scaler. Control signals transmitted via IP, RS-232, or CEC

- Front Panel Control for Menu, Auto, and Volume

- Front LED indication for PWR and signal status

- Video resolution support for 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- Dual TCP/IP and RS-232 control

- Ethernet enabled, HDBaseT extension

- Contact closure for screen control or display lift control

- Auto Display Control

- Power over Ethernet for associated TX

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- CEC support

- HDCP 1.4 support

- EDID management support

- PCM 2Ch, de-embedding analog audio out

- PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio for HDMI OUT and HDBaseT IN

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- Transmits IP and AV signals up to 328 ft. (100m)

- CAT5e/6 @ 1080p up to 100 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- 48V Input Power

- CE and FCC certified

## HDVS-150-TX Product Specifier

The 3X1 switcher / transmitter shall have two HDMI and one VGA inputs, as well as audio on 3.5 connector. The transport shall extend HDMI, VGA (including audio), RGBHV. The switching shall have input select, as well as display control buttons on the front panel. Device configuration, operation and, maintenance shall be available via Web GUI access. When two or more signal inputs are available, the transmitter shall include integrated switcher with signal sensing. The switcher shall switch to the last detected input (when not used with a control system by the same manufacturer). The 4K /UHD Input Switcher shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty

- 4K/UHD capability @ 60 Hz with 4:2:0 Chroma subsampling

- Full support of 4K/UHD streaming services and playback device

- Video resolution support for 4096×2160@24/25/30/50/60Hz*, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Signal bandwidth of 10.2 Gbps

- CEC Pass through

- HDCP 1.4 compliant - Adheres to latest specification for High-bandwidth Digital Content Protection

- PCM 2CH input audio

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- CAT5e/6 @ 4K up to 35 meters

- CAT5e/6 @ 1080p up to 60 meters

- CAT6a/7 @ 4K up to 40 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Firmware upgradable via mini USB

- CE and FCC certified

## HDVS-150-TX-WP Product Specifier

The 2X1 switcher/ transmitter shall be able to extend HDMI, VGA (including audio), RGBHV, YUV, and Composite over HDBaseT port. The 2X1 switcher / transmitter shall have input select, as well as display control on the front panel. Device configuration, operation and, maintenance shall be available via software management and Web GUI access. When two signal inputs are available, the transmitter shall include integrated switcher with signal sensing.  The switcher shall switch to the last detected input (when not used with a control system by the same manufacturer). The 4K /UHD Input Switcher shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty.

- US, 2-gang Decora-style wall plate

- 2X1 HDBaseT Switcher - One HDMI input plus a VGA input with 3.5mm audio connector

- Front panel input select button

- Front panel display on/off control

- Front LED indication for PWR and signal status

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control: Selects active input when sources are connected

- Automatic input selection using video detection technology

- EDID management support

- CEC support

- HDCP 1.4 support

- PCM 2CH input audio

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- Transmits AV signals up to 230 ft. (70m)

- CAT5e/6 @ 4K up to 35 meters

- CAT5e/6 @ 1080p up to 60 meters

- CAT6a/7 @ 4K up to 40 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- Power consumption up 18.7W (when paired)

- CE and FCC certified

## HDVS-150-RX Product Specifier

The HD Scaling receiver shall accept HDBaseT audio and video Signal, and outputs HDMI, as well as de-embedded balanced audio output, and RS-232 port. The HD scaling receiver shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty.

- HD video scaler with HDMI output and input resolution control

- Front Panel Control for Menu setup

- Front LED indication for PWR and signal status

- Video resolution support for 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- Power over Ethernet for associated TX

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- CEC support

- HDCP 1.4 support

- EDID management support

- PCM 2Ch, de-embedding analog audio out

- Master Audio for HDMI OUT and HDBaseT IN

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- Transmits IP and AV signals up to 328 ft. (100m)

- CAT5e/6 @ 1080p up to 70 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- 48 Input Power

- CE and FCC certified

## UHD-EX-100CE-RX-PSE Product Specifier

The 4K /UHD Transport System shall be an advanced signal extender system incorporating the following device specifications, and backed up with 10 years manufacture limited warranty.

- 4K/UHD capability @ 60 Hz with 4:2:0 Chroma subsampling

- Full support of 4K/UHD streaming services and playback device

- Video resolution support for 4096×2160@24/25/30/50/60Hz*, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Signal bandwidth of 10.2 Gbps

- CEC Pass through

- HDCP 2.2 compliant - Adheres to latest specification for High-bandwidth Digital Content Protection

- PoE power source – remotely powers a PoE-compatible transmitter – Adheres to IEEE 802.3af PoE Power Sourcing Equipment (PSE)

- AV, Ethernet, power, and control over HDBaseT - Receives 4K/UHD video, embedded audio, 100Base-T Ethernet, power, and control through a single cable

- Extended distance HDMI extension - Receives HDMI signals up to 330 feet (100 meters) @ 1080p with CAT5e/6 or 4K/UHD using CAT6a/7 cable

- Extends RS-232, CEC, and Ethernet control signals

- Audio format support for PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- CAT5e/6 @ 4K up to 70 meters up to 230 feet

- CAT6a/7 @ 4K up to 100 meters up to 328 feet

- CAT5e/6 @ 1080p up to 100 meters up to 328 feet

- HDMI IN/OUT @ 4K up to 5 meters up to 15 feet

- HDMI IN/OUT @ 1080p up to 10 meters up to 30 feet

- Field-updatable firmware via Front panel USB port

- Front panel Power and Link status LEDs

- 1 inch (25 mm) high quarter rack-width enclosure

- Power supply certified for CE, FCC, RoHS, cULus, RCM,CCC

## UHD-EX-100CE-TX-PD Product Specifier

The 4K /UHD Transport System shall be an advanced signal extender system incorporating the

following device specifications, and backed up with 10 years manufacture limited warranty.

- 4K/UHD capability @ 60 Hz with 4:2:0 Chroma subsampling

- Remotely powered via PoE

- Video resolution support for 4096×2160@24/25/30/50/60Hz*, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Signal bandwidth of 10.2 Gbps

- CEC Pass through

- HDCP 2.2 compliant - Adheres to latest specification for High-bandwidth Digital Content Protection

- PoE power source – remotely powers a PoE-compatible transmitter – Adheres to IEEE 802.3af PoE Power Sourcing Equipment (PSE)

- AV, Ethernet, power, and control over HDBaseT - Receives 4K/UHD video, embedded audio, 100Base-T Ethernet, power, and control through a single cable

- Extended distance HDMI extension - Receives HDMI signals up to 330 feet (100 meters) @ 1080p with CAT5e/6 or 4K/UHD using CAT6a/7 cable

- Extends RS-232, CEC, and Ethernet control signals

- Multi-channel audio compliant - Supports PCM, Dolby® Digital, Dolby Digital Plus™, Dolby TrueHD, Dolby Atmos®, DTS Digital Surround™, DTS-HD Master Audio™, and DTS:X™

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- CAT5e/6 @ 4K up to 70 meters up to 230 feet

- CAT6a/7 @ 4K up to 100 meters up to 328 feet

- CAT5e/6 @ 1080p up to 100 meters up to 328 feet

- HDMI IN/OUT @ 4K up to 5 meters up to 15 feet

- HDMI IN/OUT @ 1080p up to 10 meters up to 30 feet

- Field-updatable firmware via Front panel USB port

- Front panel Power and Link status LEDs

- 1 inch (25 mm) high quarter rack-width enclosure

- Power supply certified for CE, FCC, RoHS, cULus, RCM, CC

## AT-HDVS-300-KIT Product Specifier

The Soft Codec Conferencing System shall have ability to provide AV switching, USB and HDMI extension, plus system control using PC-based codecs such as Skype, WebEx, and Go ToMeeting. The system shall have a transmitter and a receiver with five video inputs shared between both devices for HDMI, DisplayPort, and analog video signals. The transmitter and receiver will have three USB type B connectors for host computers, in addition to two USB hubs on each of the transmitter and receiver. The system must be able to support 4k/UHD sources and displays. The following device specifications shall be backed up with 10 years manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control: Selects active input when sources are connected

- Automatic input selection using video detection technology

- EDID management support

- CEC support

- HDCP 1.4 support

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- Transmits IP and AV signals up to 328 ft. (100m)

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- Power consumption up 24W

- CE and FCC certified

- Power supply certified for CE, FCC, RoHS, cULus, RCM, CCC

## AT-UHD-SW-5000ED Product Specifier

The 5X1 4K/UHD switcher shall have two HDBaseT inputs, three HDMI inputs, mirrored HDMI and HDBaseT outputs, balanced audio outputs, automatic input selection, and automatic display control capability. It shall transmit AV signals along with Ethernet, power, and control over a single cable to displays up to 330 feet (100 meters).  For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control using Consumer Electronics Control (CEC)*, IP and RS-232

- Automatic input selection using video detection technology

- EDID management support

- CEC support

- HDCP Switchable- Compliant / non-compliant - 1.4 support

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- Transmits IP and AV signals up to 330 ft. (100m)

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- Power consumption up 24W

- CE and FCC certified

- Power supply certified for CE, FCC, Level VI, RoHS, cULus, RCM, CCC

## AT-UHD-SW-52ED Product Specifier

The 5X1 4K/UHD switcher shall have five HDMI inputs, with mirrored HDBaseT and HDMI outputs, balanced analog audio outputs, automatic input selection, and automatic display control capability. It shall transmit AV signals along with Ethernet, power, and control over a single cable to displays up to 330 feet (100 meters). For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control using Consumer Electronics Control (CEC)*, IP and RS-232

- Automatic input selection using video detection technology

- EDID management support

- CEC support

- HDCP Switchable- Compliant / non-compliant - 1.4 support

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- Transmits IP and AV signals up to 330 ft. (100m)

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

- Compatible with IP- Management System software

- Firmware upgradable via mini USB

- Power consumption up 22W

- CE and FCC certified

- Power supply certified for CE, FCC, cULus, RCM

## AT-HD-SC-500 Product Specifier

The 3x1 switcher scaler will have two HDMI inputs, VGA, as well as analog audio inputs for embedding onto HDMI output. The switcher scaler shall have Automatic display control using CEC, IP and RS-232, as well as Automatic input selection using video detection technology. The switcher scaler Shall be controlled via TCP.IP, RS-232. It will have ability to adjust volume control, input selection, and turning display power ON /OFF from the front panel. For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for up to 1920 x 1200 including 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800, 1280×768, 1152×864, 1024×768, 800×600, 640×480

- YUV and RGB Color Space

- 4:4:4, 4:2:2, 4:2:0 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control using Consumer Electronics Control (CEC)*, IP and RS-232

- Automatic input selection using video detection technology

- EDID management support

- HDCP Switchable- Compliant / non-compliant - 1.4 support

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- HDMI IN/OUT @ 1080p up to 10 meters / 30 feet

- Signal bandwidth of 6.75 Gbps

- Web GUI setup, management and support

## AT-UHD-CLSO-601 Product Specifier

The UHD/ 4K multi-format switcher shall have four HDMI inputs, two multi-function analog inputs, and a mirrored HDMI, and HDBaseT output with a built-in 4K scaler. It will have Automatic display control using CEC, IP and RS-232, as well as Automatic input selection using video detection technology. The switcher Shall be controlled via TCP.IP, RS-232. For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- Composite / S-Video formats NTSC, NTSC4, PAL, PAL-M, PAL-N, SECAM

- Automatic display control using Consumer Electronics Control (CEC)*, IP and RS-232

- Automatic input selection using video detection technology

- EDID management support

- HDCP Switchable- Compliant / non-compliant - 1.4 support

- PCM 2CH input audio

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

## AT-UHD-CLSO-612ED

The UHD/ 4K multi-format switcher shall have 2HDMI inputs, two HDBaseT inputs, two multi-function analog inputs, a mirrored HDMI, and HDBaseT output with a built-in 4K scaler. It will have Automatic display control using CEC, IP and RS-232, as well as Automatic input selection using video detection technology. The switcher Shall be controlled via TCP.IP, RS-232. For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV, RGB Color Space

- 4:4:4, 4:2:2 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- Automatic display control using Consumer Electronics Control (CEC)*, IP and RS-232

- Automatic input selection using video detection technology

- EDID management support

- HDCP Switchable- Compliant / non-compliant - 1.4 support

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

## AT-UHD-CLSO-824

The UHD/ 4K multi-format switcher shall have four HDMI inputs, three HDBaseT inputs, one multi-function analog inputs, two HDBaseT outputs as well as two HDMI mirrored outputs. It will have an audio output mixer, as well as Ethernet and RS-232 and IR HDBaseT insertion. The switcher Shall be controlled via TCP.IP, RS-232. For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV, RGB Color Space

- 4:4:4, 4:2:2 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- EDID management support

- HDCP Switchable- Compliant / non-compliant - 1.4 support

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

## AT-UHD-CLSO-840

The UHD/ 4K multi-format switcher shall have five HDMI inputs, three HDBaseT inputs, two HDBaseT outputs, as well as two discrete HDMI outputs. It will have an audio output switcher, as well as audio embedding and de-embedding. The switcher Shall be controlled via TCP.IP, RS-232. For long term protection investment, the followings product and specification shall support a 10- year manufacture limited warranty.

- Video resolution support for 4096×2160@24/25/30/50/60Hz, 3840×2160@24/25/30/50/60Hz, 2048x1080p, 1080p@23.98/24/25/29.97/30/50/59.94/60Hz, 1080i@50/59.94/60Hz, 720p@50/59.94/60Hz, 576p, 576i, 480p, 480i

- VESA resolution support for 2560×2048, 2560×1600, 2048×1536, 1920×1200, 1680×1050, 1600×1200, 1600×900, 1440×900, 1400×1050, 1366×768, 1360×768, 1280×1024, 1280×800 1280×768, 1152×768, 1024×768, 800×600, 640×480

- YUV, RGB Color Space

- 4:4:4, 4:2:2 Chroma Subsampling

- 8-bit, 10-bit, 12-bit Color Depth

- EDID management support

- HDCP 1.4 compliant

- PCM 2CH input audio

- Audio for HDMI IN and HDBaseT OUT: PCM 2Ch, LPCM 5.1, LPCM 7.1, Dolby Digital, DTS 5.1, Dolby Digital+, Dolby TrueHD, DTS-HD Master Audio, Dolby Atmos, DTS:X

- Audio sample rates of 32kHz, 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Up to 24-bit rate

- TCP/IP and RS-232 control of switcher

- Ethernet enabled, HDBaseT extension

- CAT5e/6 @ 4K up to 70 meters

- CAT5e/6 @ 1080p up to 100 meters

- CAT6a/7 @ 4K up to 100 meters

- HDMI IN/OUT @ 4K up to 5 meters

- HDMI IN/OUT @ 1080p up to 10 meters

- Signal bandwidth of 10.2 Gbps

- Web GUI setup, management and support

## OmniStream VC-2 (AT-OMNI-111, AT-OMNI-112, AT-OMNI-121, AT-OMNI-122)

- The AV over IP system shall enable the high density (up to two channels per box) of UHDp30 video at a latency of less than 0.5 video frames while being powered over either PoE or external power.

- For minor network problems, the AV over IP system shall support FEC (forward error correction).

- For major input problems, the AV over IP system shall support both physical and logical redundancy.

- For sensitive content, the AV over IP system shall support content encryption using AES128 encryption.

- For long term protection investment, the system shall support a 10 -year manufacturer limited warranty.

Note: the numbering of the items below is intended to match the OmniStream VC-2 spec document once you remove the leading 3.7 numbers from each section.

### Video Encoder and Decoder Specification

#### Video inputs/Outputs

##### Interfaces

- Dual HDMI per box (AT-OMNI-112 and AT-OMNI-122)

- Single HDMI per box (AT-OMNI-111 and AT-OMNI-121)

#### Scaling & output resolutions

- Scaling from 800x600p60 to 1080p60 or UHDp30

#### Input resolution detection

- Automatic input resolution detection

##### Encoding/Decoding formats

- VC-2 SMPTE 2042

##### Density

- Up to two video services per box (AT-OMNI-112 and AT-OMNI-122)

- Single video service per box (AT-OMNI-111 and AT-OMNI-121)

#### VC-2 specification

##### Rate control

- CBR (constant bit rate) or cVBR (capped variable bit rate)

##### Bit Rate

- 15 Mbps to 900 Mbps

##### Bit depth

- 8 bit, 10 bit, 12 bit

##### Chroma sub-sampling

- 4:2:0, 4:2:2, 4:4:4

##### Latency

- End to end latency of 0.5 frames or less

### Audio Sub System Specifications

#### Audio input/output

- HDMI audio input

- Ability to output balanced analog audio on the decoder

- Ability to re-embed balanced analog audio input on the decoder

- Ability to decode audio from a source different from video

#### Pass through and re-embedding

##### Audio formats

- LPCM 2.0, 5.1, 7.1

- DTS 5.1, DTS-HD Master Audio

- Dolby Digital AC3 2.0, 5.1

- Dolby Digital+ E-AC3 2.0, 5.1, 7.1

- Dolby TrueHD 2.0, 5.1, 7.1

##### Audio density

- Dual 2.0 balanced analog audio outputs (AT-OMNI-122)

- Single 2.0 balanced analog audio output (AT-OMNI-121)

#### Audio down mix

- Downmix of incoming LPCM to 2.0

#### Audio volume control

- Front panel volume control

- Volume control via management system

- Audio mute

### System

#### Decoder redundancy modes

- Decoder supports redundancy of incoming streams

- Support of only sending IGMP join requests when a redundancy switch occurs

- Support being always joined to both primary and backup multicast streams

##### Redundancy Events

- Physical link redundancy (primary and backup streams on different physical interfaces)

- Logical link redundancy (primary and backup streams may be on the same physical interface)

#### Streaming mode

- Video, audio, and RS232 are streamed on separate multicasts

#### IP protocols

##### IP address assignment

- IP v4

- DHCP, static, or automatic IP configuration

##### Host name

- Ability to configure device host namte

##### Discovery

- Support of mDNS/DNS-SD

##### IGMP

- IGMP v2 and IGMP v3 (SSM)

- IGMP Immediate leave

##### Streaming and multicasting

- RTP output (encoder) and RTP input (decoder)

##### SAP/SDP

- SAP/SDP including session name, description, and originator

##### Packet loss protection

- FEC (forward error correction) per SMPTE 2022-5:2013

- FEC matrix sizes from 1x4 to 20x20

##### QoS

- QoS tagging

#### RS232 protocol

##### RS232 settings

- Baud rates from 2400-115200

- Configurable parity, stop bit, and flow control

- 6 pin RS232 connector (3 pins used per service)

##### Projector/display control

- Display on/off via RS232 using customizable commands

##### RS232 over IP

- RS232 encapsulation into IP for transmission between encoder and decoder

- Ability to receive RS232 from a source different from video

- Ability to perform bidirectional RS232 communication when configured as unicast

#### Display control

##### Aspect ratio

- Support for the following aspect ratios: full screen, 16:9, 16:10, 4:3, same as source

##### Test pattern

- Test pattern generation automatically (due to loss of input) or manually

##### Input HDMI switching

- Switch between HDMI inputs using front panel buttons

##### Video output matrix

- Ability to route any input to any output

##### EDID

- EDID copy of connected displays

- EDID internal source

- Export EDID information via RS232 and management system

- External EDID insertion via management system

- Independent EDID per output

##### HDCP

- HDCP 1.4

- Hardware ready for HDCP 2.2

- HDCP modes: compliant, non-compliant, and automatic

- Automatic scrambling of the IP stream for HDCP sources

##### CEC

- CEC per HDMI 1.3a

- CEC commands: monitor on/off, input select, volume up/down

##### Display mode

- Ability to automatically turn on connected display when source signal is detected

- Ability to automatically turn off connected display when source signal is removed

#### Scrambling

- AES 128 encryption of video and audio

- Scrambling key is controlled by management system

#### Firmware upgrade

##### Field upgradability

- Remote upgrade of firmware via management system

##### Safe upgrade

- If firmware upgrade fails, the system shall revert to previous firmware

#### Configuration Import / Export

- Export of unit configuration

- Import of unit configuration

- Storage of up to 5-unit configuration files with user defined names

#### Data reporting

- The unit shall support configuration and display of description and location

#### Unit location string

- User defined location string may be up to 100 text characters

#### Power up/Reset

- Power up time is less than 20 seconds

#### Alarms and warnings

- The system shall support forwarding of alarms and warning messages to the management system

### Hardware

#### Interfaces

##### HDMI

- Dual HDMI per box (AT-OMNI-112 and AT-OMNI-122)

- Single HDMI per box (AT-OMNI-111 and AT-OMNI-121)

- HDMI 1.4

- Hardware ready for HDMI 2.0

##### Network

- Dual RJ45 per box (AT-OMNI-112 and AT-OMNI-122)

- Single RJ45 per box (AT-OMNI-111 and AT-OMNI-121)

##### Audio

- Dual 2.0 balanced input/output per box (AT-OMNI-112 and AT-OMNI-122)

- Single 2.0 balanced input/output per box (AT-OMNI-111 and AT-OMNI-121)

- Volume control (gain) – 0 dB to 15 db

- Audio mute

#### Serial communication

- 6 pin RS232 connector (3 pins used per service)

#### Front panel buttons

##### Display on/off

- Display on/off front panel buttons for controlling connected display(s)

##### Input select

- Input select front panel buttons for controlling connected display(s)

##### Volume up/down

- Volume up/down front panel buttons for controlling connected display(s)

##### Reset

- Unit reset button recessed on front panel

##### ID

- ID front panel button to indicate unit in management system

##### Push button control

- Management system can disable front panel buttons

#### LED

- HDMI status LEDs

- Network status LEDs

- Power status LED

- Management system can blink LEDs to locate unit

#### Power supply

- Power may be connected via either external PSU or PoE

- Unit can automatically switch between external PSU and PoE depending on what is connected

##### PoE

- <13W power consumption (no analog audio), so unit may be powered with one PoE

- Ability to dynamically switch which RJ45 PoE is sourced from in case of input failures

##### External PSU

- External PSU two pin captive screw phoenix connector

##### MTBF

- Unit MTBF of 90,000 hours

##### Compliance

- FCC part 15 subpart B, ICES-003, Issue 2, Class A

- EN 60950-1 safety specification

- EN 55022 and CISPR22

- EN 55024

- RoHS

#### Dimensions

- H x W x D: 34 x 208 x 112 (mm)

### Environmental

#### Temperature

- 0°C - 50°C degrees Celsius external ambient operating temperature

- -40°C to 70°C degrees’ Celsius external ambient storage (non-operational) temperature

#### Humidity

- 90% relative humidity, non-condensing

#### ESD

- ±12kV (air) and ±8kV (contact)

# Definitions and Abbreviations

- 100BaseT -An Ethernet standard for transmitting at 100 Mbps over twisted pair cable

- 1000Base-T / Gigabit Ethernet - An Ethernet standard that transmits at 1 Gbps over twisted pair cable

- 4K - Digital cinema resolution at 4092x2160 pixels. Frame rates can range from 24 to 120         fps. 4K is often mentioned along with Ultra HD, which has a video resolution of 3840x2160

- 720p - Progressive-scan HDTV transmission standard. Refers to an active pixel rate of 1280x720 with a vertical refresh rate of 60 frames per second for NTSC countries or 50 frames per second for PAL/SECAM countries

- AES/EBU - Audio Engineering Society/European Broadcasting Union

- Aspect ratio - The relationship of the horizontal dimension to the vertical dimension of an Image.

- Balanced –  3-conductor, 110-ohm cabling with an XLR connector, typically referred to as “AES/EBU audio.”

- CAT 5 - Category 5 - Describes the network cabling standard that consists of four unshielded twisted pairs of copper wire terminated by RJ-45 connectors. CAT 5 cabling supports data rates up to 100 Mbps and a minimum bandwidth of 100 MHz CAT 5 is based on the EIA/TIA 568 Commercial Building Telecommunications Wiring Standard.

- CAT 5e - Enhanced Category 5- The standard for the next higher grade of unshielded

- twisted pair - UTP beyond Category 5. The CAT 5e specification was developed to provide more robust support for 1000Base-T, and includes tighter limits than CAT 5 for NEXT, ELFEXT, and return loss. CAT 5e cabling supports a minimum bandwidth of 100 MHz

- CAT 6 - Category 6 - The standard for the next higher grade of unshielded twisted pair UTP cabling beyond CAT 5e. The standard defines components (cable and connecting hardware) and cabling (basic link and channel) for Category 6 channels, as well as Level III field tester requirements. CAT 6 cabling supports a minimum bandwidth of 250 MHz

- CAT 6a - Category 6a, or Augmented Category 6 A higher grade of twisted pair cabling beyond standard CAT 6-rated cable, with twice the bandwidth capability. CAT 6a features a shielded twisted pair cable design, with braid and foil shielding around the unshielded twisted pair conductors. This design improves crosstalk performance at high frequencies. CAT 6a cabling supports a minimum bandwidth of 500 MHz

- CAT 7 - Category 7 - The cable standard for 10 Gigabit Ethernet using shielded twisted pair - STP cable. CAT 7 features strict guidelines for crosstalk and system noise, requiring shielding for each pair of wires and the cable. CAT 7 cabling supports a minimum bandwidth of 600 MHz

- CEC -  Consumer Electronics Control

- DDC - Display Data Channel

- DHCP - Dynamic Host Configuration Protocol.

- DPCP- DisplayPort Content Protection

- DRM - Digital Rights Management

- EDID - Extended Display Identification Data

- FCC - Federal Communications Commission

- fps - Frames per Second

- HDBaseT- A signal extension technology defined by the HDBaseT Alliance for combining and transporting HDMI audio/video, Ethernet, control signals, and DC power using category-type twisted pair cable.

- HDCP -  High-bandwidth Digital Content Protection

- HDMI - High-Definition Multimedia Interface

- HDTV - High Definition Television

- IP - Internet Protocol

- JPEG - Joint Photographic Experts Group

- LAN - Local Area Network

- MAC - Media Access Control

- Matrix Switcher- A means of selecting an input source and connecting it to one or more outputs. Like a regular switcher, but with multiple inputs and multiple outputs.

- MHz - Megahertz

- Miracast -  A protocol developed by the Wi-Fi Alliance for streaming of audio and video           media between devices over a Wi-Fi connection

- Multicast - Multicast addressing is a network technology for the delivery of information to a group of destinations simultaneously using the most efficient strategy to deliver the messages over each link of the network only once, creating copies only when the links to the multiple destinations split. A single stream is sent from the source to a group of recipients

- PCM - Pulse Code Modulation

- RS-232 - An Electronic Industries Association - EIA serial digital interface standard.                      specifying the characteristics of the communication path between two devices using                           either DB-9 or DB-25 connectors.

- S/PDIF - Sony/Philips Digital Interconnect Format

- SMPTE - Society of Motion Picture and Television

- Source - AV equipment connected to the inputs of the AV switcher or extender

- STP - Shielded Twisted Pair, CATx cable with internal metallic electromagnetic shielding for individual pairs and/or the cable overall.

- TCP/IP - Transmission Control Protocol/Internet Protocol

- TMDS - Transition Minimized Differential Signaling

- TOSLINK - An optical fiber connection standard for digital audio developed by Toshiba

- Unbalanced -  2-conductor, 75-ohm coaxial cable with an RCA connector.

- Ultra HD - Video resolution at 3840x2160 pixels, with frame rates from 24 to 60 fps. Ultra            HD is often mentioned along with 4K.

- USB - Universal Serial Bus

- UTP - Unshielded Twisted Pair, CATx cable without internal metallic electromagnetic shielding

- VESA - Video Electronics Standards Association

- Y Cr Cb - Used to describe the color space for interlaced component video.

- YUV - Defines color space in terms of Y - luminance or brightness, and two color-difference components, U - red minus luminance and V - blue minus luminance. YUV is interchangeable with “Y Cr Cb” for digital component video and “Y Pb Pr” for analog component video.